WTFPOS

Per-Location Setup Topology

Prepared by: Arturo Jose T. Magno   |   Client: WTF! Corporation (Christopher Samonte, CEO)

Last updated: March 11, 2026

Each location runs one Mini PC as the SvelteKit Node server — the single source of truth for that branch. All other devices (cashier tablet, kitchen screens) are thin clients: Android tablets running Chrome browser, pointed at the mini PC over local WiFi. No data replication, no sync — every device reads and writes to the same server.

Internet is NOT required for daily operations. Internet is only needed for: owner cross-branch dashboard and cloud backup (Phase 2).

# Alta Citta — Tagbilaran (tag)

Location: Alta Citta Mall, Tagbilaran City, 6300 Bohol

Type: Retail — full POS + Kitchen + Stock  |  Tables: 20 dining tables

## Device Roles

| Device | Route | Role |
| --- | --- | --- |
| Cashier tablet | /pos | Order taking, payments, checkout |
| Dispatcher tablet | /kitchen/dispatch | Expo — oversees all stations, sides queue |
| Meat station tablet | /kitchen/weigh-station | Meat weighing via Bluetooth scale |
| Stove tablet | /kitchen/stove | Dishes and drinks queue |
| Mini PC | — | Server, RxDB, print server, cash drawer trigger |

## Network

| Device | Connection | IP |
| --- | --- | --- |
| Router | — | 192.168.1.1 |
| Mini PC | LAN cable (wired) | 192.168.1.10 (static) |
| All tablets | WiFi | 192.168.1.x (DHCP) |

## Print Flow

# Alona Beach — Panglao (pgl)

Location: Panglao Circumferential Road, Alona Beach Rd, Tawala, Panglao, 6300 Bohol

Type: Retail — full POS + Kitchen + Stock  |  Tables: 20 dining tables

Identical setup to Alta Citta. Same device roles, same network layout, same print flow.

If IP addressing changes between branches, update the static IP assignment on the mini PC and the router’s DHCP reservation table.

# Tagbilaran Central Warehouse (wh-tag)

Location: Tagbilaran City, 6300 Bohol

Type: Warehouse — inventory only (no POS, no kitchen)

Access: Stock receiving, deliveries, transfers to branches

## Device Roles

| Device | Route | Role |
| --- | --- | --- |
| Warehouse tablet | /stock/* | Receive deliveries, initiate transfers, stock counts |
| Mini PC | — | Server, RxDB, print server for transfer slips |

## What the Warehouse Does NOT Have

- No POS (/pos route hidden for wh-tag location)

- No kitchen screens

- No cash drawer

- No floor plan

# Cross-Branch Owner View

The owner’s device (any browser, any location) can see both branches via the SSE aggregate — reads live order data from both Tag and Pgl servers simultaneously. Internet connection required.

Phase 2 replaces SSE with Ably pub/sub channels for more reliable cross-branch real-time sync.